RE RESOURCE ASSESSMENT COURSE Lesson 2: Solar Resource Assessment

Lesson 2 QUIZ

- What is the solar constant?

  - Solar irradiance at a surface located at a height of 0 m above mean sea level, perpendicular to the sun rays, with an approximate value of 800 W/m2.

  - Solar irradiance at a surface located outside of the earth’s atmosphere, perpendicular to the sun rays, at mean earth-sun distance, with an approximate value of 1361 W/m2;

  - Solar irradiance at a surface located at a height of 0 m above mean sea level, with a value of 1361 W/m2;

  - Solar irradiance at a surface located outside of the earth’s atmosphere, perpendicular to the sun rays, at mean earth-sun distance, with an approximate value of 800 W/m2;

- Considering the solar radiation components, identify the correct definition of global solar radiation:

  - Solar radiation emanating from the solar disk and the circumsolar region;

  - Solar radiation scattered and reflected by the air, clouds and other particles in the atmosphere;

  - The sum of the direct and diffuse components of solar radiation;

  - Total amount of solar radiation incident in the globe.

- Solar resource assessment data should be used for:

  - Pre-feasibility studies;

  - Due-diligence and acceptance;

  - Operation and Management;

  - All of the above.

- Classify this statement: satellite derived data can be used for solar resource assessment having low uncertainties, high temporal resolutions and high spatial resolution.

  - True

  - False

- Classify this statement: ground measured data can be used for solar resource assessment having high accuracy, high temporal resolution and limited spatial resolution.

  - True

  - False

- Classify this statement: The use of solar irradiance measurements for resource assessment requires the application of quality control mechanisms.

  - True

  - False

Lesson 2 SOLUTIONS

- What is the solar constant?

Solar irradiance at a surface located outside of the earth’s atmosphere, perpendicular to the sun rays, at mean earth-sun distance, with an approximate value of 1361 W/m2

- Considering the solar radiation components, identify the correct definition of global solar radiation:

The sum of the direct and diffuse components of solar radiation

- Solar resource assessment data should be used for:

All of the above

- Classify this statement: satellite derived data can be used for solar resource assessment having low uncertainties, high temporal resolutions and high spatial resolution.

False

- Classify this statement: ground measured data can be used for solar resource assessment having high accuracy, high temporal resolution and limited spatial resolution.

True

- Classify this statement: The use of solar irradiance measurements for resource assessment requires the application of quality control mechanisms.

True